# Tuya Zigbee CK-TLSR8656 Temp-Humidity Sensor Driver Version 1.0.4 Documentation

1. Driver Overview

- Purpose: This driver enables reliable temp/humidity readings, offset tuning, and live diagnostics.

- Supported Devices: Tuya CK-TLSR8656 Zigbee sensor, tested on Hubitat Elevation C8-Pro

- Key Features:

  - Synthetic refresh logic

  - Offset recalibration

  - Timestamped attributes

  - Unit conversions (°C/°F, RH%)

2. Setup Instructions

- Installation: Import driver via Hubitat's custom driver editor

- Pairing Tips: Ensure Zigbee network is active, sensor is reset prior to pairing

- Initial Configuration:

  - Set offset values

  - Toggle logging

  - Choose units

3. Live Diagnostic Usage

- TODO: State Variables:

  - lastTempUpdate, calibrationState, sensorSilentCounter

- Event Examples:

- Temperature: 22.3°C (Raw=2218 + Offset=0.5)

- Humidity: 47% RH (Calibrated)

- TDashboard Attributes:

  - temperature, humidity

  - TODO: lastUpdated, driverVersion

4. Troubleshooting Guide

| Issue | Possible Cause | Suggested Fix |
| --- | --- | --- |
| No updates | Sensor idle | Use synthetic refresh |
| Skewed readings | Offset misconfigured | Recalibrate in preferences |
| Duplicate logs | Debug verbosity too high | TODO: Use “Basic” debug level |

5. Driver Versioning & Changelog

- TODO: Embed version string like 1.2.0 | 2025-07-20

- TODO: Include:

  - Added synthetic refresh throttling

  - Modularized calibration logic

  - Improved dashboard traceability

6. Developer Notes

- Logic breakdown:

  - Parsing → Calibration → Conversion → Emission

- Reusability targets:

  - parseTemperature(), emitHumidityEvent(), performRefresh()

- Future improvements:

  - Battery-level monitoring

  - Health check pings

⚙️ Core Structural Breakdown

1. Initialization & Metadata

- Declares device capabilities (TemperatureMeasurement, RelativeHumidityMeasurement, Sensor, Configuration, etc.)

- Sets fingerprint metadata for Tuya Zigbee TLSR8656 sensor identification

- Initializes preferences (offsets, debug logging, unit conversions)

2. Attribute Parsing & Handling

- Parses incoming Zigbee messages from sensor

  - Temperature and humidity values decoded and adjusted

  - Re-applies synthetic offsets

  - Supports both °C/°F and RH% conversions

- Attributes are timestamped and state-tracked for visibility and diagnostics

3. Synthetic Calibration & Live State

- Applies modular offset reapplication logic

- Tracks calibration state (e.g. calibrating, calibrated, notApplicable)

- Records last calibration timestamp for auditability

4. Refresh & Polling Logic

- Implements synthetic “refresh” flow for dormant sensors

  - Re-emits last known values with updated timestamp

  - Prevents stale dashboard data

- Triggers from manual refresh or scheduled polling via runIn

5. Preferences & Unit Conversion

- Modular conversion between Celsius/Fahrenheit

- Dynamic RH formatting (raw %, rounded, string-safe)

- Fully traceable: All conversions are logged and timestamped

6. Logging & Diagnostics

- Verbose debug logging with structured output

- Diagnostic state includes last updated, calibration status, and original/raw values

- Logging is toggleable via preference

7. Driver Events & Commands

- Supports configure(), refresh(), updated() lifecycle methods

- Emits platform events for temperature, humidity, and driver status